[CODEX LOGO]

Codex Platform

Raspberry Pi 5 Kiosk

Installation & Setup Guide

Version 1.0

May 2026

CONFIDENTIAL — Internal Use Only

Table of Contents

1   Overview

1.1   What This Guide Covers

1.2   How It Works

1.3   Hardware Required Per Stall

2   Pre-Installation Checklist

2.1   What You Need Before Starting

2.2   Raspberry Pi OS Installation

3   One-Command Installation

3.1   Automatic Setup

3.2   Manual Installation

4   Device Registration in Codex Admin

4.1   Register the Pi as a Device

4.2   Map Device to Stall

5   Configure the Tap Script

5.1   Edit Configuration

5.2   Multiple Stalls

6   Testing

6.1   Test NFC Reader

6.2   Test NFC Card Read

6.3   Test the Tap Script Manually

6.4   Test Consent Screen

6.5   Verify Lead in Vendor Inbox

7   Start as a Service

7.1   Enable Auto-Start

7.2   Service Commands

7.3   View Live Logs

8   Display Setup

8.1   HDMI Display Configuration

8.2   Touch Screen (Optional)

8.3   Prevent Screen Sleep

9   NFC Stickers

9.1   About the Stickers

9.2   Attaching to ID Cards

9.3   First Tap Behaviour

10   Troubleshooting

11   Event Day Checklist

12   Fleet Management

Appendix A — Complete Config Reference

Appendix B — API Endpoints Used by Pi 5

Appendix C — File Locations on Pi

Section 1 — Overview

1.1  What This Guide Covers

This guide explains how to install and configure a Raspberry Pi 5 as an NFC tap kiosk at an exhibition stall. When complete, attendees can tap their ID card or wristband on the NFC reader and immediately see a consent screen on the connected HDMI display.

This guide covers:

•  Installing Pi 5 as an NFC tap kiosk at an exhibition stall

•  Connecting ACR122U USB NFC reader to the Pi

•  Connecting an HDMI display for the consent screen

•  Configuring the Codex tap handler script

•  Testing the complete flow end to end

•  Running the service automatically on every boot

•  Monitoring the device via the Ops fleet dashboard

1.2  How It Works

The following flow shows what happens from the moment an attendee presents their ID card:

| Step | What Happens |
| --- | --- |
| Step 1 | Attendee holds ID card near the ACR122U reader |
| Step 2 | Pi reads the NFC chip UID (unique hardware identifier) |
| Step 3 | Pi sends UID to Codex API via POST /interactions/nfc-tap |
| Step 4 | API looks up or creates attendee; returns consent_url |
| Step 5 | Chromium opens the consent URL on the HDMI display |
| Step 6 | Attendee taps Accept or Decline on screen |
| Step 7 | Lead appears in the vendor inbox within seconds |

1.3  Hardware Required Per Stall

| Item | Recommended Model | Approx. Cost | Where to Buy |
| --- | --- | --- | --- |
| Raspberry Pi 5 (8 GB RAM) | Official Raspberry Pi 5 — 8 GB | Approx. £80 / ₹8,500 | raspberrypi.com |
| Power Adapter | Official Pi 5 27W USB-C Adapter | Approx. £12 / ₹1,200 | raspberrypi.com |
| MicroSD Card | 32 GB+ Class 10 or A1 (endurance card) | Approx. £8 / ₹600 | Amazon / Flipkart |
| NFC Reader | ACS ACR122U USB NFC Reader | Approx. £30 / ₹2,500 | ACS, Amazon |
| HDMI Display | Any HDMI monitor or Pi 7" touchscreen | Varies | Local supplier |
| HDMI Cable | Micro-HDMI to HDMI (Pi 5 port is micro) | Approx. £5 / ₹400 | Amazon |
| USB Keyboard + Mouse | Any USB keyboard and mouse (setup only) | Approx. £10 | Any supplier |
| NFC Stickers | NTAG213 or NTAG215 (pack of 100) | Approx. ₹10–25 each | Amazon, AliExpress |
| Internet Connection | WiFi (configured in Pi OS) or Ethernet | Existing venue WiFi | N/A |

Section 2 — Pre-Installation Checklist

2.1  What You Need Before Starting

Verify each item below before proceeding. Missing any of these will cause the setup to fail.

□  Pi 5 hardware assembled and powered on

□  Raspberry Pi OS (Bookworm, 64-bit) installed on MicroSD

□  Pi connected to the internet (WiFi or Ethernet)

□  ACR122U plugged into a USB port on the Pi

□  HDMI display connected and powered on

□  Codex admin credentials (admin@yourdomain.com and password)

□  Device ID assigned for this Pi — e.g. device-ie-01

□  Stall ID for this Pi — e.g. stall-ie-a1

□  Event ID for the active event — e.g. event-indiaexpo

2.2  Raspberry Pi OS Installation

If Raspberry Pi OS is not yet installed, follow these steps on a Mac or Windows PC:

1.  Download Raspberry Pi Imager from raspberrypi.com/software

2.  Insert the MicroSD card into your Mac or PC

3.  Open Imager → Choose Device: Raspberry Pi 5

4.  Choose OS → Raspberry Pi OS (64-bit) [Bookworm]

5.  Click the gear icon (⚙) → Enable SSH → set username: pi → set a password → configure your WiFi network name and password

6.  Click Write and wait for the card to be written and verified

7.  Insert the MicroSD card into the Pi 5 and power on

8.  Wait 2 minutes for the first boot to complete

9.  If SSH was enabled, you can connect: ssh pi@<pi-ip-address>

Tip: To find the Pi's IP address, check your router's DHCP client list or run: ping raspberrypi.local

Section 3 — One-Command Installation

3.1  Automatic Setup

Once the Pi is running and connected to the internet, open a terminal on the Pi (or SSH in from your Mac/PC) and run this single command:

curl -s https://raw.githubusercontent.com/kkarnati1980/IEXM/main/scripts/pi5/setup.sh | bash

This one command does the following automatically:

•  Updates the system package list

•  Installs libnfc-bin and libnfc-dev (NFC reader driver)

•  Installs chromium-browser (for the consent screen display)

•  Installs pcscd (PC/SC daemon for smart card support)

•  Configures ACR122U via /etc/nfc/libnfc.conf

•  Adds a USB device permission rule (no sudo needed for NFC reads)

•  Downloads codex-tap.sh to /home/pi/

•  Creates the codex-nfc systemd service (auto-starts on every boot)

•  Configures Chromium kiosk autostart on the desktop

Expected terminal output:

[setup] Codex Platform — Pi 5 Setup
[setup] Updating system packages...
[setup] Installing NFC tools...
[setup] Configuring ACR122U (libnfc)...
[setup] Adding udev rule for ACR122U...
[setup] Installing codex-tap.sh...
[setup] Creating systemd service codex-nfc...
[setup] Setup complete!

NEXT STEPS:
  1. Edit /home/pi/codex-tap.sh — fill in CONFIG
  2. Test manually: /home/pi/codex-tap.sh
  3. Start service: sudo systemctl start codex-nfc

If you see errors during any step, note the error message and refer to Section 10 (Troubleshooting).

3.2  Manual Installation (if automatic setup fails)

If the one-command setup fails, follow these steps manually. Run each command on the Pi:

Step 1 — Update system packages:

sudo apt update && sudo apt upgrade -y

Step 2 — Install required packages:

sudo apt install -y libnfc-bin libnfc-dev pcscd chromium-browser curl jq

Step 3 — Configure NFC reader:

sudo mkdir -p /etc/nfc
sudo nano /etc/nfc/libnfc.conf

# Add these lines:
allow_autoscan = true
allow_intrusive_scan = false
log_level = 1
device.name = "ACS ACR122U"
device.connstring = "usb"

Step 4 — Download the tap script:

curl -fsSL https://raw.githubusercontent.com/kkarnati1980/IEXM/main/scripts/pi5/codex-tap.sh \
  -o /home/pi/codex-tap.sh
chmod +x /home/pi/codex-tap.sh

Step 5 — Create the systemd service:

sudo nano /etc/systemd/system/codex-nfc.service

# Paste this content:
[Unit]
Description=Codex Platform NFC Tap Handler
After=network-online.target graphical.target
Wants=network-online.target

[Service]
Type=simple
User=pi
Environment=DISPLAY=:0
Environment=XAUTHORITY=/home/pi/.Xauthority
ExecStartPre=/bin/sleep 10
ExecStart=/home/pi/codex-tap.sh
Restart=always
RestartSec=10

[Install]
WantedBy=graphical.target

sudo systemctl daemon-reload
sudo systemctl enable codex-nfc

Section 4 — Device Registration in Codex Admin

4.1  Register the Pi as a Device

This step must be performed ONCE per Pi. It provisions a device authentication token that the tap script uses to authenticate with the Codex API.

Step 1 — Log in to Codex Admin:

•  URL:  https://codex-api-production-064f.up.railway.app/login

•  Email: admin@yourdomain.com  |  Password: (your admin password)

Step 2 — Navigate to Admin → Devices and verify your device appears (e.g. device-ie-01).

Step 3 — Provision a Device Token:

From a terminal on your Mac, PC, or the Pi itself, run the following commands. Replace the email and password with your admin credentials, and replace device-ie-01 with your actual device ID.

# Step A: Login as admin and capture the token
TOKEN=$(curl -s -X POST \
  https://codex-api-production-064f.up.railway.app/auth/login \
  -H "Content-Type: application/json" \
  -d '{"email":"admin@test.com","password":"TestPass123!"}' \
  | python3 -c "import sys,json; print(json.load(sys.stdin).get('token',''))")

echo "Admin token: $TOKEN"

# Step B: Provision a device credential
curl -s -X POST \
  https://codex-api-production-064f.up.railway.app/devices/device-ie-01/credentials/provision \
  -H "Content-Type: application/json" \
  -H "Authorization: Bearer $TOKEN" \
  -d '{"credential_label": "pi5-stall-a1"}' \
  | python3 -c "import sys,json; print(json.load(sys.stdin).get('bearer_token',''))"

Step 4 — Save the output:

The command prints a device token starting with dvc_... Copy it and save it securely. You will need it in Section 5. This token is shown only once.

⚠   Important: The token is shown only once at provisioning time. If you lose it, provision a new one. Old tokens can be deactivated from the admin panel.

4.2  Map Device to Stall

In the Codex admin panel, verify that the device (device-ie-01) is:

•  Assigned to the correct stall (e.g. stall-ie-a1)

•  Linked to the active event (e.g. event-indiaexpo)

•  Status shows: active

If the device is not yet mapped, go to Admin → Events → [your event] → Stalls → [your stall] → Assign Device.

Section 5 — Configure the Tap Script

5.1  Edit Configuration

Open the tap script on the Pi and edit the CONFIG section at the top:

nano /home/pi/codex-tap.sh

Find the CONFIG block and fill in your values:

# ── CONFIG — edit these values for each Pi ────────────────────────
API_BASE="https://codex-api-production-064f.up.railway.app"
DEVICE_TOKEN="dvc_xxx"         # ← paste token from Section 4
DEVICE_ID="device-ie-01"      # ← your device ID (must match Codex DB)
EVENT_ID="event-indiaexpo"    # ← active event ID
STALL_ID="stall-ie-a1"        # ← assigned stall ID
DISPLAY_NUM=":0"              # ← HDMI display number (leave as :0)
# ──────────────────────────────────────────────────────────

Save with Ctrl+X then Y then Enter.

5.2  Multiple Stalls

When setting up more than one Pi at an event, each Pi needs its own configuration. Keep these the same across all Pis:

•  API_BASE  (always the same URL)

•  EVENT_ID  (same event for all stalls)

Change these for each Pi:

•  DEVICE_TOKEN  — provision separately for each device (Section 4.1)

•  DEVICE_ID     — must match the device registered in Codex admin

•  STALL_ID      — must match the stall assigned to that Pi

Section 6 — Testing

6.1  Test NFC Reader Detection

Check that the ACR122U is recognised by the Pi:

nfc-list

Expected output:

nfc-list uses libnfc 1.8.0
Found 1 NFC device: ACS / ACR122U PICC Interface 00 00
1 ISO14443A passive target(s) found: (none if no card present)

If no device is found:

sudo systemctl restart pcscd
nfc-list
# If still missing, check USB: lsusb | grep 072f

6.2  Test NFC Card Read

Hold an NFC card, sticker, or wristband near the reader:

nfc-poll

Expected output when a card is presented:

NFC device: ACS / ACR122U opened
Waiting for a target...
Target detected!
ISO/IEC 14443A (106 kbps) target:
    ATQA (SENS_RES): 00  44
       UID (NFCID1): 04 A3 B2 C1 D4 E5 F6
      SAK (SEL_RES): 00

The UID line (04 A3 B2 C1 D4 E5 F6) is the unique card identifier. Press Ctrl+C to stop polling.

6.3  Test the Tap Script Manually

Run the tap script directly from the terminal:

/home/pi/codex-tap.sh

Hold an NFC card near the reader. Expected output:

[10:30:15] [codex] ═════════════════════════════════
[10:30:15] [codex]   Codex NFC Tap Handler
[10:30:15] [codex]   Device:  device-ie-01
[10:30:15] [codex]   Stall:   stall-ie-a1
[10:30:15] [codex]   Event:   event-indiaexpo
[10:30:15] [codex] NFC reader detected ✓
[10:30:15] [codex] Waiting for NFC card taps...
[10:30:22] [codex] Tap: UID=04A3B2C1D4E5F6
[10:30:22] [codex]   interaction_id=interaction-abc123  new=true
[10:30:22] [codex] Opened: https://...consent_token=xyz...

Chromium should open on the HDMI display showing the consent screen. Press Ctrl+C to stop the script.

6.4  Test Consent Screen

On the HDMI display, verify the consent screen shows correctly:

□  "Welcome!" message appears at the top

□  "Do you agree to share your details with this exhibitor?" question is visible

□  Green "Yes, share details" button is shown

□  "No thanks" button is shown

Tap "Yes, share details":

□  Screen shows: ✅ Thank you!

□  "Your details have been shared with the exhibitor"

□  Window closes automatically after 3 seconds

Tap "No thanks":

□  Screen shows: 👋 No problem!

□  "Your visit has been anonymously recorded"

□  Window closes automatically after 3 seconds

6.5  Verify Lead in Vendor Inbox

Login as the vendor and confirm the lead was recorded:

•  URL: https://codex-api-production-064f.up.railway.app/login

•  Go to Lead Inbox

•  The tap interaction should appear within a few seconds

•  For accepted consents: attendee details visible

•  For declined consents: anonymous visit recorded

Section 7 — Start as a Service

7.1  Enable Auto-Start

The setup script already registered the service. To start it immediately:

sudo systemctl start codex-nfc
sudo systemctl status codex-nfc

Expected status output:

● codex-nfc.service - Codex Platform NFC Tap Handler
   Loaded: loaded (/etc/systemd/system/codex-nfc.service; enabled)
   Active: active (running) since Thu 2026-05-14 09:00:00 BST; 1min ago
 Main PID: 1234 (codex-tap.sh)

The service is set to start automatically on every reboot. No manual action is needed after the initial setup.

7.2  Service Commands

| Command | Purpose |
| --- | --- |
| sudo systemctl start codex-nfc | Start the service |
| sudo systemctl stop codex-nfc | Stop the service |
| sudo systemctl restart codex-nfc | Restart (use after editing codex-tap.sh) |
| sudo systemctl status codex-nfc | Check if service is running |
| sudo systemctl enable codex-nfc | Enable auto-start on boot (already done) |
| sudo systemctl disable codex-nfc | Disable auto-start |

7.3  View Live Logs

# Live log stream (Ctrl+C to stop)
sudo journalctl -u codex-nfc -f

# Tap log file
tail -f /var/log/codex-nfc.log

Normal log output looks like:

[codex] Device: device-ie-01 | Stall: stall-ie-a1
[codex] NFC reader detected ✓
[codex] Waiting for NFC taps...
[info] Heartbeat sent (queue_depth=0)
[codex] Tap: UID=04A3B2C1D4E5F6
[codex]   interaction_id=interaction-abc123  new=true
[codex] Opened: https://...consent_token=xyz

Section 8 — Display Setup

8.1  HDMI Display Configuration

Connect the HDMI display before powering on the Pi. The Pi 5 has two micro-HDMI ports. Use port 1 (the port closest to the USB-C power connector).

If the display shows a black screen after boot:

sudo raspi-config
# Navigate to: Display Options → Screen Blanking → Disable

8.2  Touch Screen (Optional)

If using the official 7" Raspberry Pi touchscreen:

•  Connect the ribbon cable to the DSI port on the Pi 5

•  The touchscreen works as both display and input device

•  Attendees can tap Accept/Decline directly on screen — no external mouse needed

•  No additional driver installation is required with Pi OS Bookworm

8.3  Prevent Screen Sleep

To keep the consent screen visible during the event, disable screen sleep:

# Method 1: via raspi-config
sudo raspi-config
# Display Options → Screen Blanking → Disable

# Method 2: via autostart file
mkdir -p /home/pi/.config/lxsession/LXDE-pi/
nano /home/pi/.config/lxsession/LXDE-pi/autostart

# Add these lines:
@xset s off
@xset -dpms
@xset s noblank

Section 9 — NFC Stickers

9.1  About the Stickers

NFC stickers do NOT need to be programmed or encoded. The Codex platform uses the factory UID — a unique identifier built into every NFC chip at manufacture time. There is nothing to write to the sticker.

Recommended specifications:

| Property | Value | Notes |
| --- | --- | --- |
| NFC Standard | ISO 14443-A | Required for ACR122U compatibility |
| Chip Type | NTAG213 or NTAG215 | Both work; NTAG215 has more memory (unused) |
| Read Range | 3–5 cm with ACR122U | Attendee holds card near reader |
| Cost | ₹10–25 per sticker (bulk) | Order 100+ for an event |
| Where to buy | Amazon, AliExpress, NXP distributors | Search: NTAG213 NFC stickers 100 pack |

9.2  Attaching to ID Cards

Follow these steps to attach stickers to attendee badges or ID cards:

1.  Peel the NFC sticker from the backing paper

2.  Stick the sticker to the BACK of the attendee ID card (not the printed side)

3.  Press firmly for 5 seconds to ensure good adhesion

4.  Optionally: laminate over the sticker to improve durability for multi-day events

5.  Test with nfc-poll on the Pi before distributing at scale

9.3  First Tap Behaviour

Understanding how attendees are handled:

| Scenario | What Happens | API Field |
| --- | --- | --- |
| First tap of a new sticker | Codex creates a new anonymous attendee automatically | is_new_attendee: true in API response |
| Repeat tap from same sticker | Same attendee record returned | is_new_attendee: false |
| Same sticker at different stalls | Same attendee, new interaction per stall | Cross-stall visit tracking |
| Attendee accepts consent | Details shared with vendor; lead visible in inbox | Attendee profile enriched |
| Attendee declines consent | Anonymous visit recorded; no details shared | Privacy preserved |

Section 10 — Troubleshooting

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| nfc-list shows no device found | ACR122U not detected via USB | Unplug and replug USB. Run: lsusb ¦ grep 072f If missing from lsusb, try a different USB port. Then: sudo systemctl restart pcscd && nfc-list |
| "DEVICE_TOKEN not set" error in log | CONFIG section not edited | Run: nano /home/pi/codex-tap.sh Paste your dvc_xxx token into DEVICE_TOKEN Restart: sudo systemctl restart codex-nfc |
| Chromium not opening on HDMI | DISPLAY variable wrong or desktop not running | Check: echo $DISPLAY (should be :0) Ensure Pi has booted to desktop (not CLI-only). Run script from desktop terminal, not SSH |
| "Device not found" from API (404) | DEVICE_ID does not match Codex database | Check Admin → Devices in admin panel. Ensure DEVICE_ID in codex-tap.sh matches exactly. |
| "403 Forbidden" from API | Token revoked, expired, or wrong device | Provision a new token (Section 4.1). Paste into codex-tap.sh, restart service. |
| Consent screen shows but buttons do nothing | consent_token may have expired (valid 8 hrs) | A fresh NFC tap generates a new token. This is expected for stale open windows. |
| Service not starting | Configuration error in codex-tap.sh | Run manually: /home/pi/codex-tap.sh Read the error output and fix CONFIG section. |
| Taps queued every time (offline mode) | No internet connection | Check WiFi: ping google.com Reconnect Wi-Fi via raspi-config or network manager. Taps retry automatically when connection returns. |
| Black screen on HDMI display | Screen blanking enabled | sudo raspi-config → Display Options → Screen Blanking → Disable |
| Service crashes and restarts repeatedly | Script permission error or missing dependency | Check: ls -la /home/pi/codex-tap.sh (needs +x) Check: which nfc-list (should exist) Check logs: sudo journalctl -u codex-nfc -n 50 |

Section 11 — Event Day Checklist

Pre-Event (1 Hour Before Doors Open)

□  Pi powered on and service running:  sudo systemctl status codex-nfc

□  NFC reader detected:  nfc-list

□  HDMI display on and screen not sleeping

□  Test tap with one sticker — consent screen appears on display

□  Verify test lead appears in vendor inbox

□  Internet connection stable:  ping google.com

□  Log file clean:  tail /var/log/codex-nfc.log

□  Offline queue empty:  cat /var/log/codex-queue.csv (should be empty)

During Event

□  Monitor logs if issues arise:  sudo journalctl -u codex-nfc -f

□  If Pi becomes unresponsive:  sudo systemctl restart codex-nfc

□  If HDMI screen goes blank:  move mouse or press any key

□  If taps stop registering:  unplug and replug ACR122U, then restart service

Post-Event

□  Export leads from vendor dashboard before leaving

□  Stop service:  sudo systemctl stop codex-nfc

□  Backup log file:  cp /var/log/codex-nfc.log ~/event-YYYYMMDD.log

□  Check offline queue:  if not empty, retry or report to admin

□  Power off Pi safely:  sudo shutdown -h now

Section 12 — Fleet Management

12.1  Monitor All Devices

The Ops fleet dashboard shows all Pi devices in real time:

•  URL: https://codex-api-production-064f.up.railway.app/login  →  Ops → Fleet Management

| Dashboard Column | Meaning |
| --- | --- |
| Online/offline status | Green = heartbeat received in last 10 min; Red = no recent heartbeat |
| Last heartbeat time | Timestamp of most recent heartbeat from the Pi |
| NFC reader status | connected / disconnected as reported by the tap script |
| Assigned stall | Which stall and event this device is registered to |
| Local queue depth | Number of taps queued offline (0 = all synced) |

12.2  Heartbeat Schedule

Each Pi sends a heartbeat to the API every 5 minutes. The heartbeat includes:

device_id, event_id, stall_id, local_queue_depth, battery_level (always 100 for mains-powered Pi), reader_status, connectivity_status.

If a device has not sent a heartbeat for more than 10 minutes, it shows as offline in the fleet dashboard.

Appendix A — Complete Configuration Reference

All configuration variables in /home/pi/codex-tap.sh:

| Variable | Example Value | Description |
| --- | --- | --- |
| API_BASE | https://codex-api-production-064f.up.railway.app | Base URL of Codex API. Do not change. |
| DEVICE_TOKEN | dvc_xxx (from provision) | Device bearer token. Provision per Section 4.1. |
| DEVICE_ID | device-ie-01 | Must match device registered in Codex admin. |
| EVENT_ID | event-indiaexpo | Active event ID. Same for all stalls at one event. |
| STALL_ID | stall-ie-a1 | Stall ID assigned to this Pi. Unique per stall. |
| DISPLAY_NUM | :0 | X display number for Chromium. Leave as :0. |
| LOG_FILE | /var/log/codex-nfc.log | Path to tap event log file. |
| QUEUE_FILE | /var/log/codex-queue.csv | Offline retry queue for failed taps. |
| DEBOUNCE_SECONDS | 3 | Ignore same card within N seconds to avoid duplicates. |
| HEARTBEAT_INTERVAL | 300 | Seconds between heartbeat pings (5 minutes). |

Appendix B — API Endpoints Used by Pi 5

| Endpoint | Purpose | Auth Required | Key Response Fields |
| --- | --- | --- | --- |
| POST /interactions/nfc-tap | Record NFC tap, get consent URL | Device token (dvc_xxx) | interaction_id, consent_url, is_new_attendee |
| POST /device/heartbeat | Send device health ping | Device token (dvc_xxx) | Acknowledges heartbeat |
| POST /consents/capture | Record attendee consent decision | Attendee session token | consent recorded |
| POST /devices/:id/credentials/provision | Get a device bearer token | Admin token | bearer_token (shown once) |

Appendix C — File Locations on Pi

| File / Location | Purpose |
| --- | --- |
| /home/pi/codex-tap.sh | Main NFC tap handler script. Edit CONFIG section here. |
| /etc/systemd/system/codex-nfc.service | Systemd service definition. Auto-starts on boot. |
| /etc/nfc/libnfc.conf | ACR122U NFC reader configuration for libnfc. |
| /var/log/codex-nfc.log | Tap event log. Use tail -f to monitor live. |
| /var/log/codex-queue.csv | Offline tap queue. Retried automatically on reconnect. |
| /home/pi/.config/autostart/chromium-kiosk.desktop | Chromium kiosk autostart entry. |
| /etc/udev/rules.d/99-acr122u.rules | USB permission rule for ACR122U (no sudo for NFC reads). |

Codex Platform — Raspberry Pi 5 Kiosk Installation & Setup Guide  |  Version 1.0  |  May 2026

CONFIDENTIAL — For authorised technicians only  |  Support: kishore@kiot.io